# Chapter 3

# An overview of 8086 microprocessor

- Features of 8086 microprocessor

- Functional diagram of 8086 microprocessor

- Registers and Flags

- ALP Development Tools: Editor, Assembler and linker

## Features of 8086 microprocessor

## The most prominent features of an 8086 microprocessor are as follows −

- It has an instruction queue, which is capable of storing six instruction bytes from the memory resulting in faster processing.

- It was the first 16-bit processor having 16-bit ALU, 16-bit registers, internal data bus, and 16-bit external data bus resulting in faster processing.

- It is available in 3 versions based on the frequency of operation −

  - 8086 → 5MHz

  - 8086-2 → 8MHz

  - (c)8086-1 → 10 MHz

- It uses two stages of pipelining, i.e. Fetch Stage and Execute Stage, which improves performance.

- Fetch stage can prefetch up to 6 bytes of instructions and stores them in the queue.

- Execute stage executes these instructions.

- It has 256 vectored interrupts.

- It consists of 29,000 transistors.

## Comparison between 8085 & 8086 Microprocessor

- Size − 8085 is 8-bit microprocessor, whereas 8086 is 16-bit microprocessor.

- Address Bus − 8085 has 16-bit address bus while 8086 has 20-bit address bus.

- Memory − 8085 can access up to 64Kb, whereas 8086 can access up to 1 Mb of memory.

- Instruction − 8085 doesn’t have an instruction queue, whereas 8086 has an instruction queue.

- Pipelining − 8085 doesn’t support a pipelined architecture while 8086 supports a pipelined architecture.

- I/O − 8085 can address 2^8 = 256 I/O's, whereas 8086 can access 2^16 = 65,536 I/O's.

- Cost − The cost of 8085 is low whereas that of 8086 is high.

## Functional Diagram of 8086 microprocessor

8086 Microprocessor is divided into two functional units, i.e., EU (Execution Unit) and BIU (Bus Interface Unit).

### EU (Execution Unit)

Execution unit gives instructions to BIU stating from where to fetch the data and then decode and execute those instructions. Its function is to control operations on data using the instruction decoder & ALU. EU has no direct connection with system buses as shown in the above figure, it performs operations over data through BIU.

Let us now discuss the functional parts of 8086 microprocessors.

### ALU

It handles all arithmetic and logical operations, like +, −, ×, /, OR, AND, NOT operations.

### Flag Register

It is a 16-bit register that behaves like a flip-flop, i.e. it changes its status according to the result stored in the accumulator. It has 9 flags and they are divided into 2 groups − Conditional Flags and Control Flags.

### Conditional Flags

It represents the result of the last arithmetic or logical instruction executed. Following is the list of conditional flags :

- Carry flag − This flag indicates an overflow condition for arithmetic operations.

- Auxiliary flag − When an operation is performed at ALU, it results in a carry/barrow from lower nibble (i.e. D0 – D3) to upper nibble (i.e. D4 – D7), then this flag is set, i.e. carry given by D3 bit to D4 is AF flag. The processor uses this flag to perform binary to BCD conversion.

- Parity flag − This flag is used to indicate the parity of the result, i.e. when the lower order 8-bits of the result contains even number of 1’s, then the Parity Flag is set. For odd number of 1’s, the Parity Flag is reset.

- Zero flag − This flag is set to 1 when the result of arithmetic or logical operation is zero else it is set to 0.

- Sign flag − This flag holds the sign of the result, i.e. when the result of the operation is negative, then the sign flag is set to 1 else set to 0.

- Overflow flag − This flag represents the result when the system capacity is exceeded.

### Control Flags

Control flags controls the operations of the execution unit. Following is the list of control flags −

- Trap flag − It is used for single step control and allows the user to execute one instruction at a time for debugging. If it is set, then the program can be run in a single step mode.

- Interrupt flag − It is an interrupt enable/disable flag, i.e. used to allow/prohibit the interruption of a program. It is set to 1 for interrupt enabled condition and set to 0 for interrupt disabled condition.

- Direction flag − It is used in string operation. As the name suggests when it is set then string bytes are accessed from the higher memory address to the lower memory address and vice-a-versa.

### General purpose register

There are 8 general purpose registers, i.e., AH, AL, BH, BL, CH, CL, DH, and DL. These registers can be used individually to store 8-bit data and can be used in pairs to store 16bit data. The valid register pairs are AH and AL, BH and BL, CH and CL, and DH and DL. It is referred to the AX, BX, CX, and DX respectively.

- AX register − It is also known as accumulator register. It is used to store operands for arithmetic operations.

- BX register − It is used as a base register. It is used to store the starting base address of the memory area within the data segment.

- CX register − It is referred to as counter. It is used in loop instruction to store the loop counter.

- DX register − This register is used to hold I/O port address for I/O instruction.

### Stack pointer register

It is a 16-bit register, which holds the address from the start of the segment to the memory location, where a word was most recently stored on the stack.

### BIU (Bus Interface Unit)

BIU takes care of all data and addresses transfers on the buses for the EU like sending addresses, fetching instructions from the memory, reading data from the ports and the memory as well as writing data to the ports and the memory. EU has no direction connection with System Buses so this is possible with the BIU. EU and BIU are connected with the Internal Bus.

It has the following functional parts −

- Instruction queue − BIU contains the instruction queue. BIU gets up to 6 bytes of next instructions and stores them in the instruction queue. When EU executes instructions and is ready for its next instruction, then it simply reads the instruction from this instruction queue resulting in increased execution speed.

- Fetching the next instruction while the current instruction executes is called pipelining.

- Segment register − BIU has 4 segment buses, i.e. CS, DS, SS& ES. It holds the addresses of instructions and data in memory, which are used by the processor to access memory locations. It also contains 1 pointer register IP, which holds the address of the next instruction to execute by the EU.

  - CS − It stands for Code Segment. It is used for addressing a memory location in the code segment of the memory, where the executable program is stored.

  - DS − It stands for Data Segment. It consists of data used by the program andis accessed in the data segment by an offset address or the content of other register that holds the offset address.

  - SS − It stands for Stack Segment. It handles memory to store data and addresses during execution.

  - ES − It stands for Extra Segment. ES is additional data segment, which is used by the string to hold the extra destination data.

- Instruction pointer − It is a 16-bit register used to hold the address of the next instruction to be executed.

## ALP Development Tools: Editor, Assembler and linker

Developing Assembly Language Programs (ALPs) for the 8086 microprocessor involves a toolchain that helps translate human-readable assembly code into machine-executable instructions. The core components of this toolchain are the Editor, Assembler, and Linker.

Here's a breakdown of each in the context of the 8086 microprocessors:

1. Editor

An editor (or text editor) is the initial tool you use to write your assembly language program. It's essentially a software program that allows you to create and modify plain text files.

- Function: You type your 8086 assembly instructions (using mnemonics, labels, and operands) into the editor.

- Output: The editor saves your program as a source file, typically with a .asm extension (e.g., myprogram.asm). This file contains the ASCII representation of your assembly code.

- Examples: For the 8086 era, simple text editors like Notepad (on Windows) or WordStar and PC Write were commonly used. Modern IDEs (Integrated Development Environments) for assembly programming often include their own built-in editors with features like syntax highlighting.

2. Assembler

An assembler is a crucial program that translates the assembly language mnemonics and directives from your source file into their equivalent machine code (binary instructions) that the 8086 microprocessor can understand and execute. Basic functions of assembler are:

- Translation: It reads the .asm source file and converts each assembly instruction (e.g., MOV AX, 0100h, ADD BX, CX) into its corresponding binary opcode and operand values.

- Symbol Resolution: It resolves symbolic names (labels, variable names) to their actual memory addresses or offsets. This often involves two passes:

- Pass 1: Builds a "symbol table" by assigning addresses to all labels and data items.

- Pass 2: Generates the actual machine code using the addresses from the symbol table.

- Directive Processing: It processes assembler directives (e.g., DB, DW, DD for data definition, SEGMENT, PROC, ASSUME for program organization) which provide instructions to the assembler itself, but do not directly translate into machine code.

- Input: The .asm source file created by the editor.

- Output: The assembler typically generates two files:

- Object file (.obj): This file contains the machine code (binary instructions) for your program, but it's often "relocatable." This means the addresses within the object file are relative and need to be adjusted later by the linker.

- Listing file (.lst): This optional file provides a detailed listing of your assembly code alongside the generated machine code and addresses, which is very useful for debugging and understanding how your program translates.

- Examples: For the 8086, common assemblers included MASM (Microsoft Macro Assembler) and TASM (Turbo Assembler).

3. Linker

A linker is a program that combines one or more object files (generated by the assembler) and resolves any external references to create a single, executable program. Basic function of linker are:

- Combining Object Files: Large assembly programs are often divided into smaller, manageable modules (separate .asm files) for easier development, testing, and debugging. The assembler creates a separate .obj file for each module. The linker brings these individual .obj files together.

- Resolving External References: If one module calls a procedure or accesses a variable defined in another module (or in a library), the linker resolves these "external" references, ensuring that the correct memory addresses are used.

- Linking Libraries: Programs often use pre-written routines from libraries (e.g., standard input/output functions). The linker incorporates the necessary routines from these libraries into your final program.

- Relocation: The linker adjusts the relative addresses within the object files to their absolute memory locations in the final executable file.

- Generating Executable: It produces a single executable file that can be loaded into memory and run by the operating system (or directly on the hardware in a bare-metal environment).

- Input: One or more .obj files and potentially library files.

- Output:

- Executable file (.exe): This is the final program that can be run on the 8086-based system.

- Link map file (.map): This optional file provides information about the memory layout of the linked program, including the addresses of different segments and symbols.

- Examples: For MASM, the corresponding linker was typically LINK.EXE. For TASM, it was often TLINK.EXE.

In summary, the typical workflow for developing 8086 assembly language programs using these tools is:

- Write Code: Use an Editor to create your .asm source file.

- Assemble: Use an Assembler (like MASM or TASM) to convert the .asm file into an .obj object file and optionally a .lst listing file.

- Link: Use a Linker (like LINK or TLINK) to combine the .obj file(s) and any necessary libraries into a final .exe executable file.

- Execute/Debug: The .exe file can then be executed. For debugging, tools like debuggers (e.g., DEBUG.COM in DOS, or more advanced debuggers) are used to step through the program, examine registers and memory, and identify issues.